/***********************************************************************/

/*                                                                     */

/*  FILE        :rotor.c                                               */

/*  DATE        :Fri, Mar 24, 2006                                     */

/*  DESCRIPTION :Main Program                                          */

/*  CPU TYPE    :Other                                                 */

/*                                                                     */

/*  This file is generated by Renesas Project Generator (Ver.4.0).     */

/*                                                                     */

/***********************************************************************/

#include "sfr_r813.h";

static unsigned char Port0;

unsigned char serial_cnt, tr;  // tr is rotor-time counter

int  z1, z2;                   // manual or automatic input values

int db;                        // dead band ca. 3 grd

unsigned char hv_buf[20];

void delayus (unsigned int micros)

{

unsigned int i;

for (i = 0; i < micros; i++)

{

asm("nop");

}

}

void UART0_init(void){

p1 = p1 | 0x10;   // TxD0 port output

pd1 = pd1 | 0x10; // TxD0 port direction = output

pd1 = pd1 & 0xdf; // RxD0 port direction = input

u0mr = 0x05;      // UART0 transmit/receive mode

u0c0 = 0x00;      // UART0 transmit/receive control

u0rrm = 0;        // Continuous receive mode disabled

u0brg = 130-1;    // 9600 baud @20MHz

re_u0c1 = 1;      // Reception enabled

}

void sendTxd0(unsigned char data)

{

while (ti_u0c1 == 0);	//Wait for transmission buffer emty

u0tbl = data;       	// Set transmission data

te_u0c1 = 1;        	// Transmission enabled

}

unsigned char receiveRxd0 (void)

{

unsigned char data;

unsigned char dummy;

while (ir_s0ric == 0); 	//Wait for received data

ir_s0ric = 0;    		//Clear serial reception flag

data = u0rbl;     		// Get reception data

dummy = u0rbh;     		// Get error

re_u0c1 = 1;      		// Reception enabled

return data;

}

void hv_in(void)

{

unsigned char letter, i, j;

serial_cnt = 0;

letter = receiveRxd0();

if ((letter == 'A') || (letter == 'E'))  // AZ or El input in buffer

{

while (letter != 13) {                 // end of input

hv_buf[serial_cnt] = letter;

serial_cnt++;

letter = receiveRxd0();

}

hv_buf[serial_cnt] = letter;  // this is CR

}

serial_cnt++;

// send dta back

for (i=0; i<serial_cnt; i++) {

sendTxd0(hv_buf[i]);

}

// decode data to numbers z1, z2

i = 0;

while (i < serial_cnt)

{

if (hv_buf[i] == 'A')

{

z1 = 0;         // decode 4 figures

for (j=0; j< 4; j++) z1 = 10* z1 + (hv_buf[j]-48);

i = j;

}

if (hv_buf[i] == 'E')

{

z2 = 0;        // decode 4 figures

for (j=0; j< 4; j++) z2 = 10* z2 + (hv_buf[j]-48);

i = j;

}

i++;     // else never out of loop

}  // end of while loop

while (z1 > 360) { z1 = z1 -360; }

while (z2 > 180) {z2 = z2 -180; }

z1 = (z1 * 17) /6;   // upgrade to 1020

z2 = (z2 * 17) /3;   // upgrade to 1020

}

unsigned int ad_in(unsigned char ch)

{

adcon0 = 0x80 + ch;     //Port P0 group

adcon1 = 0x28;          //10-bit mode

adst = 1;               //Conversion start

while(adst == 1){}      //Wait A/D conversion

return ad;              //AD value

}

void initclock(void) {

/*-------------------------------------------------

-  Change on-chip oscillator clock to Main clock  -

-------------------------------------------------*/

prc0 = 1;           	/* Protect off */

cm13 = 1;           	/* Xin Xout */

cm15 = 1;           	/* XCIN-XCOUT drive capacity select bit : HIGH */

cm05 = 0;           	/* Xin on */

cm16 = 0;           	/* Main clock = No division mode */

cm17 = 0;

cm06 = 0;           	/* CM16 and CM17 enable */

asm("nop");         	/* Waiting for stable of oscillation */

asm("nop");

asm("nop");

asm("nop");

ocd2 = 0;           	/* Main clock change */

prc0 = 0;           	/* Protect on */

}

void initport(void) {

Port0 = 0xff;

p0 = 0xfd;

prc2 = 1;		//protect off

pd0 = 0x09; 	//input of AD p0,1-2 ch,6-5   p0,4-7  ch,4-0

}

void initmot(void) {

prc1 = 1;

pd1 = 0x0F;   // set port 1,0 - 1,3  as bit out

pd3 = 0x00;   // set port 3,0 - 3,3  as bit in

}

void main(void)

{

int w;

unsigned int t1, t2;

initclock();

initport();

initmot();

UART0_init();  //Test -Init und ABCDEF

z1 = 512;    // midpoint AZ

z2 = 512;    // midpoint EL

tr = 10000;  // rotor time

db = 10;     // dead band ca. 3 grd

while  (1)

{

if (p3_0 == 0)             // check manual / automatic

{   z1 = ad_in(1);       // manual input

z2 = ad_in(0);

}

else

{

hv_in();         //automatic from satelite program

}

t1 = 0;

w = ad_in(6);                        // rotor AZ

if (w < (z1-db)) { p1_0 = 1;

p1_1 = 0;

t1 = z1 - w;

}

if (w > (z1+db)) { p1_0 = 0;

p1_1 = 1;

t1 = w - z1;

}

t2 = 0;

w = ad_in(5);                      // rotor EL

if (w < (z2-db)) { p1_2 = 1;

p1_3 = 0;

t2 = z2 - w;

}

if (w > (z2+db)) { p1_2 = 0;

p1_3 = 1;

t2 = w - z2;

}

if ((t2 > 0) || (t1 > t2)) {    // calulate time

t2 = t2 * tr;

delayus(t2);

}

if ((t1 > 0) || (t2 > t1)) {   // calculate time

t1 = t1 * tr;

delayus(t1);

}

p1_0 = 0;       // reset output to zero

p1_1 = 0;

p1_2 = 0;

p1_3 = 0;

delayus (50000);    // delay

delayus (50000);

}

}