Notes on Recent Additions:

ENHANCED 6800 FAMILY TABLE TO SUPPORT 68HC11

Use the '-x7' (or just 'x') command line option to enable the 68HC11

instructions.    TASM deviates from standard Motorola syntax for the

BSET,  BRSET,  BCLR,  and BRCLR instructions.   TASM requires commas

separating  all arguments.   Motorola assemblers use white space  to

separate the last one or two arguments for these instructions.  Here

are examples of each applicable instruction:

TASM                            MOTOROLA

----------------------          --------------------

BCLR    addr1,Y,bmsk            BCLR    addr1,Y bmsk

BCLR    addr1,X,bmsk            BCLR    addr1,X bmsk

BCLR    addr1  ,bmsk            BCLR    addr1   bmsk

BSET    addr1,Y,bmsk            BSET    addr1,Y bmsk

BSET    addr1,X,bmsk            BSET    addr1,X bmsk

BSET    addr1  ,bmsk            BSET    addr1   bmsk

BRCLR   addr1,Y,bmsk,lab1       BRCLR   addr1,Y bmsk lab1

BRCLR   addr1,X,bmsk,lab1       BRCLR   addr1,X bmsk lab1

BRCLR   addr1  ,bmsk,lab1       BRCLR   addr1   bmsk lab1

BRSET   addr1,Y,bmsk,lab1       BRSET   addr1,Y bmsk lab1

BRSET   addr1,X,bmsk,lab1       BRSET   addr1,X bmsk lab1

BRSET   addr1  ,bmsk,lab1       BRSET   addr1   bmsk lab1

SAMPLE FILES FOR THE SMALL C COMPILER (68HC11)

TASM  can  be  used  in conjunction with the public  domain  Small  C

Compiler  by  J.  Hendrix (as published in Doctor  Dobb's  Journal).

Unarchive the MISC.ZOO file to access the files of interest:

booz misc.zoo

Refer to the README.C11 file for an explanation of the files.

IMPROVED ERROR REPORTING

All error messages now contain file and line number information.

The format is slightly modified (from pre 2.9.3 releases of TASM)

so as to be compatible with the BRIEF editor.  The BRIEF editor

is capable of performing an assembly from within the editor and

allowing the user to scan to each occurance of an error in the

source file.

ECHO DIRECTIVE

The ECHO directive can be used to send output to the console. It can

accept a string or an expression, but only one such on each

invocation.  Consider the following example:

.ECHO "The size of the table is "

.ECHO (table_end - table_start +1)

.ECHO " bytes long.\n"

This would result in a single line of output something like this:

The size of the table is 196 bytes long.